# Egypt policy-options matrix (Table 2) – rebuilt ad hoc calculations: methodology note v0.2

Workbook: AdHocCalculations_Rebuild_v0.2.xlsx (builder build_adhoc_rebuild_v0_2.py, verification recalc_and_check_adhoc_v0_2.py).

v0.2 changes (vs v0.1). (1) Emission factors fc / fp / np / no = Egypt CBAM EF v0.1 (EmissionFactors/EGY_CBAM_EF_v0.1.xlsx, Products D:G), selected by the new switch EFSet (EGY_EF_V01 default; PROTOTYPE = v0.11 kernel EFs, kept on Inputs E columns R:U and forced in PROTOTYPE mode). Conventions: urea np = 0 (CBAM rule; v0.11 −0.733, floored to 0 in G anyway); ammonium nitrate no = integrated HNO₃ N₂O 0.79 × 1.2588 = 0.9944 (v0.11 0.97). (2) AN process ER100 = N₂O-weighted blend of the TASK‑D unabated (0.7210) and abated (0.3292) rows at the EF workbook's 50 % abatement share (3.5 / 1.25 kg N₂O per t HNO₃): central 0.6179, 2030 0.5539, long-run 0.6818 (v0.1: unabated row). (3) E_base 2030 = 62.42 Mt (v0.1 61.16). The kernel prototype adopts the same EFs and β in CPAT_Industry_Kernel_Egypt_v0.15. All assumptions sit on Inputs; PolicyMatrix keeps the original layout and is fully live. One switch (Mode) selects REBUILD (coherent method, default) or PROTOTYPE (reproduces CPAT_Industry_Kernel_Egypt_v0.11 exactly). Reporting year Yr = 2030, carbon price τ = CPAT cptraj.2 of the bundle's scenario ($20 in 2030 for all six bundles).

## 1. Inputs (Inputs tab)

| Block | Content | Source |
| --- | --- | --- |
| A Switches | Mode, Conv (CBAM convention: NOPHASE / FULL / SCALED), Yr, SigmaEff (fund shadow price σ = 20 $/t), BetaSet, BfScen, ThetaOther, IppuOther, KappaMode | this note |
| B Parameters | ε_U = ε_F = −0.5 → s_int = ε_F(1+ε_U)/(ε_U+ε_F(1+ε_U)) = 1/3; ε_Q = −0.5; P_EU = 100 $/t; b_f (derived); CBAM phase-in factor | TechNote App. A; prototype v0.11 |
| C CPAT values | 22 CPAT Outputs codes × EG1–EG4, looked up live for Yr (GHG, IPPU, energy CO2, industry CO2, price, effective price, revenues, deaths) + 8 derived rows | CPAT_Outputs sheet (trimmed CPAT Outputs 2022–2041) |
| D Bundles | code → CPAT scenario, scope (ALL/IND), process flag, θ (OBR share), φ (fund share), τ | TechNote §Bundles |
| E Products | 8 CBAM products: Q₀ (2024), g, EU exports X, price P, fuel factor F = fc+fp, process factor G = max(np,0)+max(no,0), ER100 → β = −ln(1−ER100)/100 | activity / prices: prototype v0.11 Manual inputs; EFs: Egypt CBAM EF v0.1 (EFSet); IPCC β set (TASK‑D) |
| F Year vectors | CBAM phase-in, flat $20 path, CPAT EG3 ramp (live) | EU CBAM; CPAT |

Bundle mapping: 1A→EG1 (ALL, process priced), 2A→EG1 (ALL, fuels only), 2B→EG2 (ALL, fuels only, household recycling), 3A→EG3 (IND, process priced), 3B→EG3 + θ=1 (output-based rebate), 3C→EG3 + φ=1 (abatement fund).

## 2. Core equations

CBAM block (per product i, sheet CBAM_Products; prototype v0.11 equations)

Prices: τ_p = τ·flag; OBR rate = θτ; fund price σ_eff = φσ (0 in PROTOTYPE).

Cost per tonne: k_f = τF_i; k_p = τ_p G_i; rebate m_i = min(θτ,τ)F_i + min(θτ,τ_p)·flag·G_i; Δp_i = (k_f+k_p−m_i)/P_i.

Output: Q_i^b = Q₀(1+g)^(Yr−2024); Q_i = Q_i^b(1+Δp_i)^ε_Q.

Emissions at baseline intensity: E_f = Q_iF_i/1000, E_p = Q_iG_i/1000 (Mt).

Intensity responses: ER_f = E_f(x_f−1) with x_f = exp(b_f(τ+σ_eff)) [REBUILD; x_f = 1 in PROTOTYPE]; ER_p = −E_p(1−exp(−β_i(τ_p+σ_eff))).

Post-response emissions: E_i = E_f+ER_f+E_p+ER_p; output-channel reduction emrq_i = (E_f+E_p)(1−(1+Δp_i)^(−ε_Q)) (process part uses E_p only).

Revenue: rev_i = (E_f+ER_f)τ + (E_p·1[G_i>0]+ER_p)τ_p ($m); rebate_i = m_iQ_i/1000.

CBAM: intensity EI_i = 1000E_i/Q_i; deductible carbon price d_i = 1000rev_i/Q_i − m_i; obligation per tonne obl_i = max(0, CBF·P_EU·EI_i − S·max(0,d_i)); cbobl = ΣX_i(Q_i/Q₀)obl_i/1000; cbobl₀ = ΣX_i(1+g)^t·CBF·P_EU(F_i+G_i)/1000. Conventions: NOPHASE (default) CBF=1, S=1; FULL CBF=0.485 (2030), S=1; SCALED CBF=S=0.485.

Block metrics: E_base = ΣQ_i^b(F_i+G_i)/1000 (= 62.42 Mt in 2030; 61.16 Mt with prototype EFs); cbcov = Σcovered/Σ(E_f+E_p); cbintch = ΣE_i/Σ(E_f+E_p) − 1; cbqch = Σemrq/E_base; emrt = Σemrq + ΣER_f + ΣER_p; cbobchu = ΣX_i·obl_i / ΣX_i·CBF·P_EU(F_i+G_i) − 1.

National quantities (CPAT, scenario s of the bundle, year Yr): ΔGHG, ΔIPPU, ΔEnergyCO2, ΔIndCO2 = policy − baseline; κ = min(1, (eff.cptraj/cptraj)·EnergyCO2₀/IndCO2₀) = share of industrial energy CO2 actually priced in the CPAT run (κ(EG3) = 0.54; κ = 1 for EG1/EG2); b_f = s_int·ln(IndCO2₁/IndCO2₀)/τ on EG1 = −0.0025 per $/t.

Table-2 metrics (sheet Results; PolicyMatrix column in brackets)

Coverage [J] = (E_f⁰ + flag·E_p^base)/GHG₀, with E_f⁰ = EnergyCO2₀ (ALL) or κ·IndCO2₀ (IND).

Total reduction [K] = ΔGHG − ΔIPPU + IPPU_other + ΣER_p + Σemrq_proc + D_obr + F_fund, where IPPU_other = 0 (switch IppuOther; CPAT's IPPU scaling with industrial fuel use is removed, MajorIssues #17), D_obr = −(1−s_int)·w·ΔIndCO2 if θ>0 (OBR neutralises the output share of CPAT's industrial fuel response; w = E_f^base/(κ·IndCO2₀) = 0.57 for EG3 unless ThetaOther = 1), F_fund = κ·IndCO2₁(exp(b_fσ_eff)−1) if φ>0 (fund buys fuel-intensity abatement outside the block).

[L] = K/GHG₀. [M] = cbcov. [N] = cbintch. [O] = cbobchu (selected convention; FULL shown in AQ).

Revenue [P] = (1−φ)·(P_gross − rebates), P_gross = CPAT carbon-tax receipts on fuels (six fuels, post-response) + Σrev_p/1000; rebates = Σrebate_i/1000 (+ τ(κ·IndCO2₁ − ΣE_f,post)/1000 if ThetaOther = 1). Fund budget = φ·(P_gross − rebates); outlay upper bound = σ|ΣER_f+ΣER_p+F_fund|/1000 (3C: 0.19 vs budget 1.47 $bn).

Deaths [Q] = CPAT deaths avoided(s) × (ΔEnergyCO2 + D_obr + F_fund)/ΔEnergyCO2 for IND bundles (energy CO2 only); [R] = Q/baseline deaths.

[T] = emrt/E_base; [U] = cbqch; [AR] = CPAT Δnet new revenue(s) − (P_gross − P).

Decomposition columns AF–AM on PolicyMatrix: ΔGHG, −ΔIPPU, IPPU_other, ΣER_p, Σemrq_proc, D_obr, F_fund, check (= 0).

## 3. Results, 2030 (Mode = REBUILD, Conv = NOPHASE, EFSet = EGY_EF_V01)

v0.1 rebuilt K for reference (prototype EFs): 1A −31.6, 2A −27.7, 2B −29.3, 3A −18.0, 3B −12.8, 3C −22.4. "proto" = kernel v0.11 with its own EFs.

|  | 1A | 2A | 2B | 3A | 3B | 3C |
| --- | --- | --- | --- | --- | --- | --- |
| J coverage, % GHG — rebuilt / original / Table 2 | 63 / 72 / 72 | 57 / 65 / 65 | 57 / 65 / 65 | 14 / 24 / 20 | 14 / 24 / 20 | 14 / 24 / 20 |
| K reduction, Mt — rebuilt / original | −31.8 / −41.6 | −27.8 / −38.9 | −29.3 / −41.0 | −18.2 / −21.5 | −12.7 / −18.1 | −22.6 / −24.5 |
| L, % of GHG — rebuilt / original | −5.3 / −8.5 | −4.7 / −8.0 | −4.9 / −8.4 | −3.1 / −4.4 | −2.1 / −3.7 | −3.8 / −5.0 |
| M CBAM coverage — rebuilt / original / proto | 100 / 100 / 100 | 44.2 / 44.5 / 44.5 | 44.2 / 44.5 / 44.5 | 100 / 100 / 100 | 100 / 100 / 100 | 100 / 100 / 100 |
| N intensity, % — rebuilt / original / proto | −6.6 / −7.6 / −1.1 | −2.2 / −5.5 / 0 | −2.2 / −5.5 / 0 | −6.6 / −7.6 / −1.1 | −6.6 / −7.6 / −1.1 | −12.7 / −13.1 / −1.1 |
| O obligation per unit, % — rebuilt / original / proto NOPHASE (FULL) | −24.8 / −26.1 / −21.1 (−42.1) | −12.5 / −13.9 / −10.9 (−22.6) | −12.5 / −13.9 / −10.9 (−22.6) | −24.8 / −26.1 / −21.1 (−42.1) | −6.0 / −7.6 / −1.4 (−1.4) | −29.3 / −30.4 / −21.1 (−42.1) |
| P revenue, $bn — rebuilt / original / Table 2 | 6.83 / 5.77 / 5.8 | 6.23 / 5.18 / 5.2 | 6.19 / 5.18 / 5.2 | 1.53 / 2.38 / 1.1 | 0.32 / 0 / 0 | 0 / 0 / 0 |
| Q deaths avoided — rebuilt / original | 1564 / 1656 | 1564 / 1564 | 1631 / 1631 | 546 / 546 | 410 / 345 | 633 / 621 |
| T block emissions, % — rebuilt / original / proto | −12.3 / −15.3 / −6.8 | −4.7 / −13.4 / −2.4 | −4.7 / −13.4 / −2.4 | −12.3 / −18.1 / −6.8 | −6.6 / −15.3 / −1.1 | −18.0 / −20.6 / −6.8 |
| U block output, % — rebuilt / original / proto | −6.1 / −8.3 / −5.8 | −2.6 / −8.3 / −2.4 | −2.6 / −8.3 / −2.4 | −6.1 / −11.4 / −5.8 | 0 / −8.3 / 0 | −6.1 / −8.7 / −5.8 |
| AR Δnet revenue, $bn — rebuilt / original | 10.43 / 10.62 | 10.43 / 10.62 | 10.62 / 10.62 | 2.69 / 2.69 | 1.44 / 0 | 1.21 / – |

Verification: Mode = PROTOTYPE reproduces all 21 block metrics × 6 bundles of v0.11 to < 1e‑7 for FULL and NOPHASE (Comparison §2, Checks); REBUILD matches an independent Python re-implementation (534/534 values).

## 4. Differences

(a) vs the original AdHocCalculations.xlsb

One base: every % uses CPAT 2030 baseline total GHG (594.2 Mt); the original mixed 2026/2030 and three denominators (#4, #11).

Coverage computed, not typed; κ makes the partial industry coverage of the EG3 run explicit (J_int memo = 20.7% at κ = 1) (#12, #13).

CBAM base bottom-up (61.2 Mt from eight products) instead of 53.6 Mt (#16); process response via β and the output channel, CPAT's proportional IPPU scaling removed (#14, #17).

1A on EG1 (was EG2-based) (#3); 3B from EG3 with an explicit OBR correction instead of −13.592·(2/3)/0.5 (#9, #15); output change per bundle, not 3B's −8.3% for all (#18).

Revenue = CPAT fuel receipts (post-response) + block process revenue, net of rebates/fund; 3B now raises 0.33 $bn because only CBAM producers are rebated (ThetaOther = 0) (#10).

Deaths from the bundle's own scenario, rescaled by energy CO2 only (#2, #19); AR from the bundle's own scenario (#6); price start 2028 (#5); nothing hard-typed (#7); EG4 excluded (#8). Net effect: K is 15–30% smaller than the original for the economy-wide bundles (the original's K included CPAT's IPPU scaling and an untraceable process term) and 10–30% smaller for the industry bundles.

(b) vs the prototype kernel v0.11

Fuel-intensity channel x_f = exp(b_f(τ+σ)) (prototype: fuel intensity fixed) → N, T, O larger in magnitude.

IPCC-based β set (prototype set available via BetaSet) → ER_p 4× larger.

Fund (3C) modelled as shadow price σ = 20 $/t on both intensity channels (prototype: fund inactive) → 3C ≠ 3A.

National totals, coverage, revenue, deaths and AR from CPAT (outside the prototype's scope); OBR rebates deducted from revenue.

NOPHASE as default convention for O (prototype headline uses FULL; both shown).

Kernel v0.13/v0.14 (after this note's PROTOTYPE calibration on v0.11): 3B rebate capped by sector shares (0.97 vs 1.22 $bn), 3C fund active (shadow price 150 $/t), and Task M adds the prototype's own Table 2 (Table2_Industry). Full comparison of Table 2 vs this rebuild vs v0.14: ResultsComparison_Table2_v0.1.md.

## 5. Format changes vs the original PolicyMatrix

Assumptions moved to Inputs; new sheets CPAT_Outputs, CBAM_Products, Results, Comparison, Issues resolved, Checks; GHG tab not rebuilt.

Columns AF–AQ redefined (yellow headers): legacy half-elasticity block replaced by the K decomposition (AF–AM), export-weighted intensity change (AN), gross revenue (AO), rebates (AP), O under FULL (AQ).

Row 4 mode indicator; C3 is a formula; G7 label corrected ("$20 flat from 2028").

Rows 14–22 (non-modelled bundles) copied verbatim and greyed, not recomputed.

Trajectory rows 26–29 rebuilt as contiguous years 2027–2034 in F–M, live, plus a CBAM phase-in row.

Columns B, D, E hidden as in the original; widths and header styling preserved.

## 6. Open flags for review

κ(EG3) = 0.54: the industry CPAT run prices only part of industrial energy CO2 (aluminium, other manufacturing excluded) — re-run EG3 with full coverage, then set KappaMode = ONE.

3B revenue depends on ThetaOther (0: CBAM producers rebated only, P = 0.33 $bn; 1: all covered industry rebated, P ≈ 0).

Non-CBAM IPPU (43% of IPPU) assumed unresponsive to a fuel-only price (IppuOther).

Deaths rescaling for 3B/3C is proportional to energy CO2 (approximation).

β set (IPCC central) and σ = 20 $/t are judgements; fund outlay bound (0.19 $bn) is far below the 3C budget (1.47 $bn).

O convention: NOPHASE default vs FULL; both reported.