Marginal Abatement Cost Curves for Process Emissions in CBAM Goods

Audit-trail method, intensity reduction at USD 100/tCO₂e, and derived semi-elasticities

September 2026. Working draft for verification.

# Bottom line

At USD 100/tCO₂e, process-emissions intensity falls about 85–90% for ammonia and ammonium nitrate, about 25–35% for BOF steel, EAF steel and cement, and only 5–13% for urea and primary aluminium. The implied semi-elasticities run from about 0.05% to 2.2% per $1/t.

Which report. IPCC AR6 WGIII (2022) remains the latest IPCC mitigation assessment. AR7 is underway, with its Synthesis Report due by late 2029.

What is and is not sourced. AR6 Chapter 11 does not provide product-level MACCs. Its headline finding is that industrial mitigation costs sit roughly in the USD 50–150/tCO₂e band, with wide variation inside and outside it. The Section 11.4.1.5 cost text was not retrieved directly for this draft. Each product curve is therefore built bottom-up: option costs from IPCC where available, technology literature elsewhere, and assumptions labeled as such.

# Method

- Boundary. Process emissions only, following the IPCC 2006 industrial-process categories, per tonne of product. Combustion and indirect emissions are excluded.

- Baseline intensity. The process intensity I₀ for each good is shown in the ledger.

- Options. For each option i, record sᵢ (technical share of the remaining process emissions), its cost range, and fᵢ (fraction of the cost range at or below $100). Where a cost range straddles $100, apply IPCC AR6 Chapter 12’s convention: options spanning several cost bins are assumed evenly distributed across them.

- Cumulate multiplicatively. r(100) = 1 − Πᵢ(1 − fᵢ·sᵢ). This avoids double counting.

- Semi-elasticity. Assume I(p) = I₀·e^(−βp). Then β = −ln(1 − r)/100, reported as % change in intensity per $1/t. The point elasticity at price p is ε = −βp. Because β is proportional, it can be applied to country-specific intensities.

Horizon and cost definition. Near-term (about 2030), limited by retrofits and plant turnover. Costs follow IPCC’s definition: net lifetime costs of avoided emissions relative to a reference technology.

# Audit ledger

| Good | Baseline process intensity I₀ (t/t) | Option at or below $100 | Cost basis | Effective share |
| --- | --- | --- | --- | --- |
| BOF steel | ~1.2–1.6 (blast-furnace reductant) [assumption] | More scrap in BOF charge | Low; scrap-limited [assumption] | 0.10 |
|  |  | Gas or H₂ injection in blast furnace | ~$50–150, half counted [assumption] | 0.05 |
|  |  | CCS on blast-furnace gases: 50% capture × 30% of fleet | Verify: IEAGHG 2018-TR03 | 0.15 |
| EAF steel | ~0.05–0.15 | Bio-carbon for charge and foaming carbon (60%) | ~$50–150, half counted [assumption] | 0.30 |
|  |  | Electrode and lime efficiency | Below $0 [assumption] | 0.05 |
| Ammonia (gas) | ~1.2–1.3 pure CO₂ stream, of ~1.7 total (IPCC 2006 Tier 1) | Capture of concentrated stream (95%) × storage access (90%) | $10–40 (IPCC AR6 WGIII 11.3.6) | 0.855 |
| Urea | 0.73 (CO₂ bound in urea, 44/60) | No real option if bound CO₂ counts as emitted | — | 0–0.05 |
| AN | ~2.0: N₂O ~1.5 + ammonia CO₂ ~0.53 | N₂O catalysts (90% of 1.5) | ~EUR 1–4/tCO₂e (incl. MRV) | 1.35 t |
|  |  | Ammonia CO₂ capture (0.855 of 0.53) | $10–40 | 0.45 t |
| Aluminium | ~1.8: anode CO₂ ~1.5 + PFC ~0.3 | Anode-effect control (60% of PFC) | Negative to low | 0.18 t |
|  |  | Anode carbon efficiency (3% of 1.5) | Low | 0.045 t |
| Cement | ~0.37 (0.52/t clinker × 0.72 clinker ratio) | Clinker ratio 0.72 → 0.55 | ≤$50, often negative [assumption] | 0.24 |
|  |  | Post-combustion CCS: 90% capture, 16% of cost range ≤$100 | $88–161 (IEAGHG) | 0.15 |

Evidence anchors

- Steel sequencing (IPCC). Decarbonisation starts with retrofits for partial fuel switching plus CCU and CCS, followed by high-capture CCS or hydrogen direct reduction.

- EAF floor. Lime and graphite alone imply a lower bound of about 0.03 tCO₂/t steel.

- Ammonia (IPCC). CCS on concentrated CO₂ streams such as SMR hydrogen costs USD 10–40/tCO₂e sequestered.

- AN removal. Catalysts remove up to 99% of N₂O (tertiary) and 95% (secondary).

- AN cost. Abatement estimated at EUR 1–2 per credit, with monitoring and verification adding EUR 1–2.

- Aluminium. Inert anodes excluded as still pre-commercial.

- Cement (IPCC). Cementitious substitution can cut calcination emissions by up to 50%; calcination is 60% of emissions in best-practice plants.

- Cement CCS. Post-combustion capture at new plants: about US$161/t avoided (1 Mt/yr, Europe) and US$88/t (3 Mt/yr, Asia). Oxyfuel is cheaper but needs further R&D, so it is excluded near-term.

# Results at USD 100/tCO₂e

| Good | r(100) central [range] | β, % per $1/t central [range] | ε at $100 |
| --- | --- | --- | --- |
| Steel BOF | 27% [10–50] | 0.31 [0.11–0.69] | −0.31 |
| Steel EAF | 33% [10–50] | 0.41 [0.11–0.69] | −0.41 |
| Ammonia | 85% [50–95] | 1.90 [0.69–3.00] | −1.90 |
| Urea | 5% [0–15] | 0.05 [0–0.16] | −0.05 |
| Ammonium nitrate | 89% [50–95] | 2.21 [0.69–3.00] | −2.21 |
| Aluminium | 13% [3–25] | 0.13 [0.03–0.29] | −0.13 |
| Cement | 35% [20–60] | 0.43 [0.22–0.92] | −0.43 |

Worked example (cement): r = 1 − (1 − 0.24)(1 − 0.15) = 0.354; β = −ln(0.646)/100 = 0.0044 per $, i.e. 0.43% per $1/t.

# Flags before use

- Concrete is not a CBAM good. CBAM covers clinker and cement, so concrete is mapped to cement. Concrete-level material efficiency (IPCC’s 24–50%) reduces demand, not intensity; place it in the demand elasticity to avoid double counting.

- Clinker as a separate good. No benefit from lower clinker ratios, so r(100) is CCS-only, roughly 15%.

- Urea is accounting-dependent. Under the IPCC inventory convention, CO₂ bound in urea is netted at the plant, so plant-level process emissions are near zero. If bound CO₂ counts as emitted, capture at the ammonia plant does not help.

- AN is baseline-dependent. Plants already running secondary catalysts (common in the EU) sit near the bottom of the range; most of the effect comes from unabated N₂O.

- N₂O GWP. AR6 GWP100 = 273 with the IPCC 2006 default of 7 kg N₂O/t HNO₃ (medium-pressure plants). For EU ETS alignment, substitute their GWP value.

- Functional form is a smoothing. Ammonia and AN have near-step MACCs, so β is an arc value between $0 and $100. Cap intensity at the technical floor rather than extrapolating above $100.

- Verify before citing. BOF CCS cost, BOF and aluminium baselines, and the PFC share should be checked against IEAGHG 2018-TR03, IAI data and CBAM default values. Costs are roughly 2016–2022 dollars.

# Sources

- IPCC AR6 WGIII Chapter 11, Industry (Bashmakov, Nilsson et al., 2022): https://www.ipcc.ch/report/ar6/wg3/chapter/chapter-11/

- IPCC AR6 WGIII Chapter 12, Cross-sectoral perspectives (cost-bin distribution convention): https://www.ipcc.ch/report/ar6/wg3/chapter/chapter-12/

- IPCC AR6 WGIII Figure SPM.7 (cost definitions): https://www.ipcc.ch/report/ar6/wg3/figures/summary-for-policymakers/figure-spm-7/

- IPCC AR7 timeline: https://www.ipcc.ch/assessment-report/ar7/

- IEAGHG, CO₂ Capture in the Cement Industry (2008/3): https://ieaghg.org/publications/co2-capture-in-the-cement-industry/

- IEAGHG, Cost of CO₂ Capture in the Industrial Sector: Cement and Iron and Steel (2018-TR03): https://publications.ieaghg.org/technicalreports/2018-TR03%20Cost%20of%20CO2%20capture%20in%20the%20industrial%20sector%20cement%20and%20iron%20and%20steel%20industries.pdf

- SEI, Industrial N₂O Projects Under the CDM: Nitric Acid Production (2010): https://www.sei.org/mediamanager/documents/Publications/Climate/sei-nitricacid-9nov2010.pdf

- Clariant, EnviCat N₂O-S catalyst (2024): https://www.clariant.com/en/Corporate/News/2024/12/Clariant-EnviCattrade-N2OS-catalyst-expected-to-cutnbsp275-KTY-CO2eq-at-Sichuan-Lutianhua-nitric-aci

- Steel CCS and hydrogen in IAM modeling (Int. J. Greenhouse Gas Control, 2023): https://www.sciencedirect.com/science/article/pii/S1750583623001330

- NRDC, The Role of Inert Anodes in Aluminum Decarbonization (2026): https://www.nrdc.org/bio/ian-wells/role-inert-anodes-aluminum-decarbonization

- IPCC 2006 Guidelines, Vol. 3 IPPU (default factors; verify tables): https://www.ipcc-nggip.iges.or.jp/public/2006gl/vol3.html