# Egypt carbon pricing and CBAM: methodology

## 1. Approach

Each scenario is a CPAT run plus changes made outside CPAT. CPAT gives the national picture. The CBAM goods (steel, cement clinker, ammonia, urea, ammonium nitrate, aluminium) are modelled in the Excel model, because CPAT has no output, process-emission or trade detail. CPAT’s own change in these sectors is removed and replaced by the separate model, so nothing is counted twice.

## 2. Scenarios

|  | CPAT run | What is priced | Use of revenue |
| --- | --- | --- | --- |
| 1A | EG1 | All energy CO₂ + CBAM process emissions | Public investment |
| 2A | EG1 | All energy CO₂ | Public investment |
| 2B | EG2 | All energy CO₂ | Per-person transfers |
| 3A | EG3 | Industrial energy CO₂ + CBAM process emissions | Households |
| 3B | EG3 | As 3A, with free allowances per unit of output | Returned to producers |
| 3C | EG3 | As 3A | Abatement fund |

Price paths: EG1 and EG2 are USD 20 flat from 2028. EG3 is USD 5 in 2028, rising to USD 50 in 2034 (USD 20 in 2030).

## 3. What is taken from CPAT

Fuel use falls when fuel gets dearer. CPAT splits this into less use (lower output) and better efficiency (less fuel per unit).

With CPAT’s default elasticities, one third of the industry fuel response is efficiency and two thirds is lower output. We use the efficiency third for fuel per tonne of CBAM goods (about −5%).

We also take CPAT’s total emission change, energy CO₂, carbon tax receipts and air-pollution deaths avoided.

CPAT has no industrial output model, no trade model, no technology switching, and applies long-run responses within one year.

CPAT grows industrial process emissions in line with industrial energy CO₂. For the CBAM goods this is removed and replaced.

## 4. The CBAM goods model

Emissions per tonne are split into four types:

Fuel burned for heat.

Fuel used as a raw material or reducing agent (for example natural gas in ammonia, or gas used to make iron).

Process CO₂ from chemical reactions (limestone, electrodes, anodes).

Other process gases: N₂O from nitric acid and PFCs from aluminium.

Fuel emissions are priced in every scenario. Process emissions are priced only where the scenario says so (1A, 3A, 3B, 3C). Electricity is excluded.

For each product the model then works out, in order:

Cost. Carbon price × priced emissions per tonne, less any free-allowance rebate (3B). Divided by the product price, this is the cost increase.

Output. Output = baseline output × (1 + net cost increase)^(−0.5). Only the net cost (after rebate) affects output.

Fuel efficiency. Fuel per tonne falls by the efficiency response, based on the full price (including the 3C fund).

Process clean-up. Process emissions per tonne fall by 1 − exp(−β × price). β is the response to price for each product (section 6).

Totals. Emissions, coverage and revenue are calculated after these responses.

Baseline output grows at CPAT’s 2024–2030 sector growth (steel ×0.976, cement ×1.001). Fertiliser and aluminium follow total industry growth, as CPAT has no energy for them in Egypt.

## 5. CBAM bill

The CBAM bill for a tonne of product is: share of emissions charged × EU carbon price × emissions per tonne. We report how much this changes per tonne exported, weighting products by their 2024 exports to the EU. The EU price, the phase-in share (48.5% in 2030) and export volumes are held fixed, so they cancel out. The result is just the change in emissions per tonne of export:

Change = Σ (exports × emissions per tonne, with policy) ÷ Σ (exports × emissions per tonne, baseline) − 1

We take no credit for the carbon price paid in Egypt. This keeps the figure independent of the EU price and consistent across scenarios. A version with that credit is kept in the workbook for reference only. Calculation: Egypt_CBAM_Obligations.xlsx.

Why the scenarios differ: in 1A, 3A and 3B fuel and process emissions are both priced, so emissions per tonne fall about 5%. In 2A and 2B only fuel is priced, so they fall about 3%. In 3C the fund pays for extra clean-up, so they fall about 21%.

## 6. Main assumptions

| Assumption | Value | Basis | Confidence |
| --- | --- | --- | --- |
| Emission factors | See table below | Egypt plant types, IPCC and CBAM rules | Medium–low |
| Process response (β) | About 0.1–1.3% per USD, depending on product | IPCC AR6 cost data, in 2024 dollars | Medium–low |
| Fuel efficiency response | One third of CPAT industry response | CPAT default elasticities | Default |
| Output response | −0.5 | Placeholder | Low |
| EU carbon price; phase-in | USD 100; 48.5% (2030) | Placeholder; cancels out of the result | — |
| Production, exports, prices | 2024 data | Steel, fertiliser and trade statistics | Medium |

Emission factors used (tonnes CO₂e per tonne of product, own plant, without electricity):

| Product | Fuel heat | Fuel as input | Process CO₂ | Other gases | Total |
| --- | --- | --- | --- | --- | --- |
| Steel, DRI-EAF | 0.18 | 0.39 | 0.04 | 0 | 0.61 |
| Steel, scrap-EAF | 0.05 | 0 | 0.04 | 0 | 0.09 |
| Cement clinker | 0.31 | 0 | 0.54 | 0 | 0.85 |
| Ammonia | 0.71 | 1.26 | 0 | 0 | 1.98 |
| Urea | 0.11 | 0 | 0 | 0 | 0.11 (1.24 with ammonia) |
| Ammonium nitrate | 0.11 | 0 | 0 | 0.99 | 1.11 (with nitric acid) |
| Aluminium | 0.12 | 0 | 1.62 | 0.77 | 2.52 |

Notes: following the CBAM rule, CO₂ bound in urea is counted at the ammonia plant. Blast-furnace steel is kept for reference only.

## 7. Limits

Long-run responses are applied in a single year.

Output is inferred, not modelled. Treat output cuts as an upper bound.

There is no trade or leakage model.

Process responses use global cost data, not Egyptian plant studies.

Emission factors are fleet averages. Ammonia gas use, nitric acid N₂O control, aluminium PFC rates and the kiln fuel mix should be checked with plant data.

Scenarios 3A–3C use a CPAT run that priced only 54% of industrial energy CO₂. A full-coverage run is still needed.

Revenue is simple revenue and ignores effects on existing taxes and subsidies.