# Egypt carbon pricing and CBAM exposure: integrated methodology

## 1. Purpose

The work assesses six policy scenarios for Egypt, with CBAM-sector industry treated in detail. Each scenario is a CPAT scenario plus adjustments made outside CPAT.

| Code | Scenario | CPAT run | What is priced | Use of revenue |
| --- | --- | --- | --- | --- |
| 1A | Comprehensive levy | EG1 | Energy CO₂, all sectors + CBAM process emissions | Public investment |
| 2A | Upstream levy | EG1 | Energy CO₂, all sectors | Public investment |
| 2B | Upstream levy | EG2 | Energy CO₂, all sectors | Per-capita transfers |
| 3A | Downstream industry price | EG3 | Industrial energy CO₂ + CBAM process emissions | Households |
| 3B | as 3A + free allocation for all covered industry | EG3 + output-based rebate θ = 1 | as 3A | Returned to all covered industry per unit of output |
| 3C | as 3A + abatement fund | EG3 + fund φ = 1 | as 3A | Fund for industrial abatement |

Price paths: EG1/EG2 USD 20 flat from 2028; EG3 USD 5 in 2028 rising to USD 50 in 2034 (USD 20 in 2030).

## 2. Scope

Outputs reported (Table 2):

coverage (% of national GHG);

total emission reduction (Mt, and % of GHG);

coverage of CBAM-sector emissions;

change in CBAM-sector emission intensity;

change in CBAM obligations per tonne exported (embedded emissions per tonne, EU-export weighted);

carbon revenue;

air-pollution deaths avoided;

change in output of CBAM goods.

CBAM goods covered (eight products and routes, kernel Manual inputs rows 30–37):

steel by three routes: DRI-EAF, scrap-EAF, and BF-BOF (BF-BOF is a reference only);

grey clinker;

ammonia;

urea;

ammonium nitrate (AN);

primary aluminium.

## 3. CPAT: what is used and how it is read

Fuel demand. Fuel use F in sector *j*, fuel *k* responds to GDP *Y* and to the retail price *P*. *P* includes the carbon charge τ × EF × coverage. The response has two margins: a usage elasticity ε_U and an efficiency elasticity ε_F, with rebound acting on the efficiency term. To a close approximation, policy fuel use relative to baseline is

F₁/F₀ ≈ (P₁/P₀)^(ε_U + ε_F(1+ε_U)) × (Y₁/Y₀)^η

Energy CO₂ = F × EF, with the EF held fixed. Industrial IPPU is projected by CPAT at the growth rate of industrial energy CO₂ (CPAT Team 2024, §3.5.5.1). This is the proportional IPPU row that this method removes.

Output inferred from the usage margin. CPAT has no industrial output variable. The usage term is read as the output response, and the efficiency term as the response of energy intensity. The intensity share of the fuel response is

s_int = ε_F(1+ε_U) / (ε_U + ε_F(1+ε_U))

With ε_U = ε_F = −0.5, s_int = 1/3 and the output share is 2/3. The implied fuel-intensity semi-elasticity on EG1 is b_f = s_int · ln(IndCO₂₁/IndCO₂₀)/τ = −0.0025 per $/t.

Other CPAT quantities used (the scenario's own scenario, 2030):

ΔGHG;

ΔIPPU;

ΔEnergy CO₂ and ΔIndustry CO₂;

carbon-tax receipts by fuel (after the demand response);

deaths avoided;

Δ net new revenue.

Non-price policies. These enter as a *shadow price* on the efficiency term only. The fund (3C) uses this mechanism, applied *after* the carbon price (CPAT Team 2024, §3.1.8.1).

Known CPAT limits that matter here. CPAT has:

no explicit output model;

long-run elasticities that take full effect within one year;

no technology switches such as CCS or H₂-DRI;

no trade model, so CBAM exposure is computed outside CPAT;

fixed world fuel prices;

default IIASA emission factors that include some process emissions, corrected only at the aggregate level (§3.1.8.6).

## 4. The CBAM product block

For each product *i* (one row per product; equations as in the kernel and the rebuild CBAM_Products sheet), with τ the carbon price on fuels and τ_p = τ × (process flag) the price on process emissions:

4.1 Emission factors (Appendix A). Direct emissions per tonne are split four ways:

fc: fuel combustion;

fp: fuel used as reductant or feedstock;

np: non-fuel process CO₂ (carbonates, electrodes, anodes);

no: non-CO₂ gases (N₂O, PFC).

Each category has its own coverage switch for each scenario (kernel Scenarios M:P). Upstream levies price the fuel group F = fc + fp. Downstream charges can also price the process group G = max(np,0) + max(no,0).

4.2 Costs and rebates.

Fuel cost: k_f = τF_i.

Process cost: k_p = τ_p G_i.

Output-based rebate: m_i = min(θτ, τ)F_i + min(θτ, τ_p)·flag·G_i.

Net cost increase: Δp_i = (k_f + k_p − m_i)/P_i, where P_i is the product price.

4.3 Output response (Task H). Q_i = Q_i^b (1 + Δp_i)^ε_Q, with Q_i^b = Q₀(1+g)^(t−2024) and ε_Q by product: cement −0.10, steel and fertilisers −0.40, aluminium −0.50 (Section 5). Only the *net* price reaches output. Free allocation therefore weakens the output response but not the intensity response.

4.4 Intensity responses. These are applied to the *full* price, plus the fund shadow price σ_eff = φσ with σ = 20 $/t:

Fuel group. x_f = exp(b_f(τ + σ_eff)), so ER_f = E_f(x_f − 1). This is the CPAT efficiency margin.

Process group. ER_p = −E_p(1 − exp(−β_i(τ_p + σ_eff))), where β_i is the process semi-elasticity (Appendix B). In the kernel this is generalised to ER(P) = ER_max(1 − e^(−βP)), separately for np and no.

4.5 Emissions, revenue and CBAM metrics.

Emissions. E_f = Q_i F_i/1000 and E_p = Q_i G_i/1000 (Mt). The post-policy total is E_i = E_f + ER_f + E_p + ER_p.

Revenue. rev_i = (E_f + ER_f)τ + (E_p + ER_p)τ_p, measured after the response (fixes MajorIssues #4 and #10). Rebates are deducted.

Embedded intensity. EI_i = 1000E_i/Q_i.

Coverage. cbcov = Σcovered/Σ(E_f + E_p).

Intensity change. cbintch = ΣE_i/Σ(E_f + E_p) − 1. An EU-export-weighted variant is also reported (cbintchx).

CBAM obligation (Table 2, row O). With an unchanged EU price, the change in the obligation per tonne exported equals the change in embedded emissions per tonne exported: O = Σ X_i·EI_i(policy) / Σ X_i·EI_i(baseline) − 1, where X_i is 2024 EU exports of product i and EI_i is fuel plus process emissions per tonne. The EU price and the CBAM phase-in factor cancel. No deduction for the Egyptian carbon price is applied, so O depends only on CBAM-product intensity and differs from N only in weighting (exports rather than output). The calculation is live in the kernel workbook, sheet Table2_Final, section E.

Kernel memo (Task L). The kernel also computes an obligation net of a deduction for the domestic carbon price: d_i = 1000·rev_i/Q_i − m_i; obl_i = max(0, CBF·P_EU·EI_i − S·max(0, d_i)); total Σ X_i(Q_i/Q_i,0)·obl_i (cbobchu). P_EU (USD 100, placeholder), CBF (0.485 in 2030) and S are set by the selector FULL (2030 phase-in, CBAM factor 0.485; the default), NOPHASE (no phase-in, factor 1) or SCALED (phase-in applied to the obligation and to the deduction). These variants are memo items only and are not reported in Table 2.

4.6 Channels kept separate (no double counting).

| Channel | Driver | Acts on |
| --- | --- | --- |
| Output | net price (after rebate) | Q, hence all four categories |
| Fuel intensity | full price + fund shadow price | fc + fp, via b_f |
| Process intensity | process price + fund shadow price | np, no via β (Appendix B) |
| CPAT's IPPU row | — | removed (IppuOther = NONE) |

4.7 Table 2: composition of the final results (CBAM carve-out).

Principle. Every Table 2 result is the original CPAT run except the CBAM block. CPAT's implied change in the block, r × B, is removed and replaced by the block calculation of 4.1–4.5. Runs: 1A and 2A = EG1, 2B = EG2, 3A–3C = EG3, as run (EG3 prices 54% of industrial energy CO₂, κ = 0.537; no scaling to full coverage).

Emission cut. K = ΔGHG(run) − r × B + dB. r is the CPAT industry energy CO₂ change of the run; B is the block's baseline fuel plus process emissions, growth-matched to CPAT sector energy CO₂; dB is the block output response, the process abatement (where charged) and CPAT's fuel-intensity response i_f = (1 + r)^(s_int) − 1 applied to block fuel (3A–3C use the EG1 value: same USD 20/t, block fully priced). CPAT's proportional IPPU row is kept for non-block IPPU.

3B rebate scope. The output-based rebate covers all covered industry. In non-block industry the output share of CPAT's response, NB = ΔInd + ΔIPPU − r × B, is removed: K changes by −(1 − s_int) × NB (+7.2 Mt for EG3), the fuel part of that term enters the deaths adjustment, and the rebate τ × (κ × IND1 − block post-policy fuel CO₂) (USD 0.33bn) is deducted from revenue. There is no feedback onto CPAT receipts. Switch: kernel Manual inputs E112 (1 = all covered industry, 0 = CBAM block only).

Other columns. Revenue P = CPAT receipts of the run + τ × block fuel adjustment + block process fees − rebates − fund. Deaths Q = CPAT deaths × (ΔEnergy CO₂ + block fuel adjustment (+ 3B term)) / ΔEnergy CO₂. Coverage J = energy CO₂ in scope (EG3: κ × industry) + block process emissions where charged. N, M and T are block metrics; O is as in 4.5.

Confirmation. Kernel sheet CarveOut_Table2 computes the above for the active scenario; sheet Table2_Final holds the six scenarios, the figures printed in the final documents and their differences (all zero), and recomputes row O live.

## 5. Parameters and their basis

| Parameter | Value | Basis | Confidence |
| --- | --- | --- | --- |
| Emission factors fc / fp / np / no | Appendix A | Egypt fleet + IPCC 2006 / CBAM Annex VIII | Tier C/D; VERIFY list |
| Process semi-elasticities β_i | Appendix B; loaded in the kernel and the rebuild (AN 0.6179 blend; Al 0.2164 on np and no; NH₃/urea 0); P* = USD 122 (USD2024) | IPCC AR6 WGIII Tables 11.3, 12.3; AR4 for N₂O and PFC | Medium–low; range = 2030 vs long run |
| Usage / efficiency elasticities | CPAT Data_Elast | Labandeira et al. (2017) | CPAT default |
| s_int | 1/3 | ε_U = ε_F = −0.5 | Judgement |
| b_f | −0.0025 per $/t | s_int × CPAT EG1 industry CO₂ response | Derived |
| ε_Q (output) | Cement −0.10; steel, fertilisers −0.40; aluminium −0.50 | Product demand × pass-through × trade exposure. Cement demand −0.02 to −0.16 with pass-through 0.2–0.4 (EC / CE Delft–Oeko 2016); steel demand −0.2 to −0.3, pass-through about 0.5; Armington elasticities about 3–4 (GTAP); EU ETS firm studies find no detectable fall in output (Colmer et al. 2025). Only cement matters at USD 20/t: its carbon cost is 15% of its price, against 1–3% for the other goods. | Low–Medium |
| P_EU; CBF | USD 100; 0.485 (2030) | Placeholder; memo only (cancels out of Table 2 O) | Medium |
| σ (fund shadow price) | USD 20/t in the rebuild; solved in the kernel from a fund equal to post-abatement block payments | Judgement (rebuild fund outlay well below the budget of USD 1.48bn) | Low |
| Production, exports, prices | Kernel Manual inputs | worldsteel, IFA, trade data (2024) | Medium (T3 to formalise) |

Overall limitations:

Long-run elasticities are applied within a single year.

Industrial output is inferred, not modelled; read it as an upper bound.

There is no trade or leakage model.

Process responses come from global IPCC cost data rather than Egyptian MACCs.

National EFs are fleet averages, not plant distributions.

Revenue is "simple revenue", excluding effects on existing taxes and subsidies.

# Appendix A. Emission factors for Egypt's CBAM goods, split four ways

Source: EmissionFactors\EGY_CBAM_EF_Methodology_v0.1.md and workbook EGY_CBAM_EF_v0.1.xlsx. The workbook is built by build_ef_v0_1.py and checked by recalc_and_check.py: 0 errors, 14 checks OK.

### A.1 Method in brief

Boundary. Direct (Scope 1) emissions of the installation making the good, per tonne, using CBAM boundaries:

crude steel at the melt shop;

clinker at the kiln;

the ammonia complex;

the urea plant;

the AN plant;

primary aluminium.

Electricity (Scope 2) is excluded. A chain EF adds precursor emissions category by category (urea 0.570 t NH₃/t; AN 0.79 t HNO₃ + 0.215 t NH₃/t; HNO₃ 0.27 t NH₃/t).

Attribution. The four categories are additive: EF = fc + fp + np + no. The split attributes the same carbon to different categories and never changes the total.

fc vs fp, the "purpose of the carbon" rule. Fuel carbon charged into the process as reductant or feedstock is fp. This covers ammonia reformer feed gas, DRI reducing gas, and BF coke and PCI. Fuel burned for heat is fc. Carbon retained in DRI is carried into the EAF and counted as fp there.

np is computed stoichiometrically: CaO × 0.785 and MgO × 1.092 tCO₂/t, with a kiln-dust correction. Electrode, charge-carbon and anode carbon use CBAM carbon contents.

no = kg gas/t × GWP. The CBAM/EU MRR set (AR5) is used: N₂O 265, CF₄ 6630, C₂F₆ 11100.

Conventions.

Urea: under the CBAM rule, CO₂ bound in urea is counted at the ammonia plant, so urea np = 0. The switch urea_conv = IPCC reproduces the inventory netting of −0.733.

AN: HNO₃ is treated as a precursor, and its N₂O is shown in the chain EF.

Data hierarchy. Inputs are graded in tiers:

A: plant data (none yet);

B: Egyptian statistics;

C: technology default calibrated to Egypt's plant fleet;

D: IPCC or international default.

Every input row records its value, a low–high range, its tier, the source ID and a VERIFY flag.

### A.2 Key equations (tCO₂e/t; EF_NG = 0.0561 tCO₂/GJ; C→CO₂ = 3.664)

DRI (Midrex/HYL, Ezz Steel):

fp = NG 10.5 GJ × feed share 0.75 × EF_NG − C_DRI × 3.664 = 0.372;

fc = 10.5 × 0.25 × EF_NG = 0.147;

the carbon balance closes (check 1).

DRI-EAF steel = 0.98 × DRI + EAF:

EAF fc = 0.6 GJ × EF_NG;

EAF fp = DRI carbon − steel carbon;

EAF np = electrodes + charge carbon + limestone.

Clinker:

fc = 3.5 GJ/t × fuel-mix EF 0.0896 (coal 45 / petcoke 40 / NG 5 / HFO 2 / alternative fuels 8%) = 0.314;

np = (0.65 × 0.785 + 0.015 × 1.092) × 1.02 = 0.537;

cement memo at clinker ratio 0.85: 0.723.

Ammonia:

fp = feed 22.5 GJ × EF_NG = 1.262;

fc = (35.2 − 22.5) GJ × EF_NG = 0.712.

Nitric acid: no = [0.5 × 2.5 + 0.5 × 7.0] kg N₂O × 265/1000 = 1.259 per t HNO₃.

Aluminium:

np = anode 0.44 × (1 − 0.024) × 3.664 + baking 0.05 = 1.623;

no = (CF₄ 0.10 × 6630 + C₂F₆ 0.01 × 11100)/1000 = 0.774;

fc = 2.2 GJ × EF_NG = 0.123.

### A.3 Results (own process, CBAM conventions, tCO₂e/t)

| Good (kernel row) | fc | fp | np | no | Own | Chain |
| --- | --- | --- | --- | --- | --- | --- |
| DRI-EAF steel (30) | 0.178 | 0.393 | 0.038 | 0 | 0.609 | 0.609 |
| Scrap-EAF steel (31) | 0.045 | 0 | 0.044 | 0 | 0.089 | 0.089 |
| BF-BOF steel, reference (32) | 0.168 | 1.257 | 0.053 | 0 | 1.478 | 1.478 |
| Grey clinker (33) | 0.314 | 0 | 0.537 | 0 | 0.851 | 0.851 |
| Ammonia (34) | 0.712 | 1.262 | 0 | 0 | 1.975 | 1.975 |
| Urea (35) | 0.112 | 0 | 0 | 0 | 0.112 | 1.238 |
| Ammonium nitrate (36) | 0.112 | 0 | 0 | 0 | 0.112 | 1.953 |
| Primary aluminium (37) | 0.123 | 0 | 1.623 | 0.774 | 2.521 | 2.521 |

The kernel uses these fc / fp / np / no values (Manual inputs H30:K37), with two conventions: urea np = 0 (as here) and AN no = 0.9944 (HNO₃ N₂O 0.79 × 1.2588 folded into own emissions, so AN own = 1.107).

### A.4 Checks

The workbook runs 14 checks:

Structural: carbon balance, fuel shares, bounds.

Integrity: totals equal the sum of components; no negative components.

Plausibility against IPCC Tier 1:

ammonia 1.694–2.104;

clinker np 0.50–0.55;

aluminium np 1.45–1.75;

DRI.

Information: reconciliation of about 50.7 MtCO₂e at kernel production volumes, of which clinker is about 42.6, to be compared with the BTR inventory.

### A.5 How the split is used

fc and fp form the fuel group, which is priced by upstream and downstream levies. np and no form the process group, which is priced only by downstream or process charges. Each category is the base for one response channel (§4.6 and Appendix B.5).

### A.6 VERIFY list (in order of impact)

| # | Input | Current value | Sensitivity of the EF |
| --- | --- | --- | --- |
| 1 | Ammonia gas use | 35.2 GJ/t | ±0.056 per GJ |
| 2 | HNO₃ N₂O abatement share (Abu Qir CDM) | 50% | ±0.6 per t HNO₃ |
| 3 | Egyptalum PFC rates | — | up to +2.0 |
| 4 | Kiln fuel mix | — | coal↔NG swap ≈ 0.13 |
| 5 | DRI gas use and feed/fuel split | — | — |
| 6 | Closure of EISCO's blast furnace (unconfirmed) | — | — |
| 7 | EU default values | — | — |

# Appendix B. Process-emission semi-elasticities

### B.1 What is being estimated

The semi-elasticity β is the proportional change in process-emission *intensity* (tCO₂e per tonne of product) per USD 1/t of carbon price. Output effects are excluded; they are handled by ε_Q.

Scope is IPCC category 2 (IPPU) emissions:

calcination CO₂;

reductant and feedstock CO₂ (BF, DRI, SMR shift);

electrode, anode and flux CO₂;

N₂O from nitric acid;

aluminium PFCs.

Combustion is excluded.

### B.2 Method in brief

Evidence. IPCC AR6 WGIII publishes costs and potentials, not elasticities. Two tables are used:

Table 11.3: technology GHG reductions and cost ranges, USD2019;

Table 12.3: 2030 economic potentials by cost bin.

For N₂O and PFCs, IPCC AR4 WGIII Ch. 7 is used, cross-checked against NACAG.

Cost-bucket MACC by product. Each abatement option *i* has:

a technical reduction r_i;

a share s_i of the process pool it addresses;

a cost range [c_low, c_high];

a deployment factor d_i,h for horizon h.

Plants' costs are assumed uniform over the range, so the share of plants in the money at price τ is f_i(τ) = clip((τ − c_low)/(c_high − c_low), 0, 1). Options combine multiplicatively, which avoids double counting:

a_h(τ) = 1 − Π_i (1 − r_i s_i d_i,h f_i(τ))

Two horizons.

2030: d calibrated to the IPCC Table 12.3 potentials (constrained by deployment).

Long run: full turnover of the capital stock at Table 11.3 costs.

Central = the mean of the two. It is the recommended value for 2030–2045 policy runs. The 2030 value is the low case and the long-run value the high case.

Semi-elasticity form. Iso-semi-elastic: E(τ) = E₀ e^(−βτ), with β = −ln(1 − a(100))/100. This is the kernel convention: E stored positive, per $. It keeps emissions positive and is defined at τ = 0. A constant elasticity form is not.

The kernel uses ER(P) = ER_max(1 − e^(−βP)). With ER_max = 1 (Option A) this is identical to the iso-semi-elastic form.

Option B fits a saturating form (ER_max = A) to a(100) and a(200).

Prices are in USD2019; USD 100 (2019) ≈ USD 122 (2024). The kernel price paths must be deflated, or ER rescaled, before use.

### B.3 Results (central; ER = reduction in process intensity at USD 100)

| Product | ER at $100: 2030 / long run / central | β central (per $; kernel convention) | Option B: A / β |
| --- | --- | --- | --- |
| DRI-EAF steel | 0.113 / 0.573 / 0.343 | 0.00420 | 0.408 / 0.0184 |
| Scrap-EAF steel | 0.095 / 0.267 / 0.181 | 0.00200 | 0.399 / 0.0060 |
| BF-BOF steel (reference) | 0.102 / 0.459 / 0.280 | 0.00329 | 0.387 / 0.0129 |
| Grey clinker | 0.193 / 0.478 / 0.335 | 0.00408 | 0.433 / 0.0149 |
| Ammonia | 0.150 / 0.445 / 0.297 | 0.00353 | 0.443 / 0.0111 |
| Urea | 0.048 / 0.184 / 0.116 | 0.00123 | fit degenerate; use Option A |
| AN, unabated N₂O | 0.676 / 0.766 / 0.721 | 0.01277 | 0.739 / 0.0371 |
| AN, N₂O ≈ 90% abated | — / — / 0.329 | 0.00399 | 0.414 / 0.0158 |
| Primary aluminium | 0.069 / 0.364 / 0.216 | 0.00244 | 0.372 / 0.0087 |

For aluminium, the central value splits into np 0.128 (inert anodes) and no 0.108 (PFC / anode-effect control).

At USD 20, the central values imply intensity cuts of about 2–8% for most goods and about 23% for unabated AN.

Cross-checks:

The IPCC Table 12.3 aggregate of process options at or below USD 100 is 15–17% of 2030 process emissions (β ≈ 0.0017).

EU ETS ex-post studies find −7 to −16% at ≤ USD 50. These are total emissions dominated by combustion, so they are not transferable.

### B.4 Egypt-specific points (qualitative; the same values are used for all countries)

DRI. Concentrated DRI top-gas CO₂ is cheap to capture, but Egypt has no CO₂ transport or storage. This argues for the 2030 column in the near term.

Cement. Cement dominates process emissions. Clinker substitution (LC3; limited slag and fly ash) is the main lever.

Nitric acid. The status of N₂O abatement at the nitric acid plants (CDM-era projects at Abu Qir) decides between the unabated and abated AN rows. This item is shared with VERIFY item #2 in A.6.

Urea. Under the CBAM rule, bound CO₂ is counted at the ammonia stage. Urea's response is therefore inherited from ammonia and should not be applied twice.

# Appendix C. Glossary

| Term | Meaning |
| --- | --- |
| fc / fp / np / no | fuel combustion / fuel-based process (reductant, feedstock) / non-fuel process CO₂ / non-CO₂ process gases |
| β | process semi-elasticity, fraction of intensity per USD/t |
| ER(P) | emission-intensity reduction at price P |
| s_int | intensity share of the fuel response |
| b_f | fuel-intensity semi-elasticity |
| κ | priced share of industrial energy CO₂ in the CPAT run |
| θ | output-based rebate share |
| φ | fund share |
| σ | fund shadow price |
| ε_Q | output elasticity |
| CBF | CBAM phase-in factor |
| P_EU | EU allowance price |
| NOPHASE / FULL / SCALED | kernel memo obligation conventions (§4.5); not used in Table 2 |
| EG1–EG4 | CPAT Egypt runs |
| 1A–3C | policy scenarios |